Multi-Robot Navigation Pipeline

A Complete Technical Guide: APF + NH-ORCA + Integration

Architecture: Hybrid A* → APF → Intention Blending → NH-ORCA → Diff-Drive Kinematics

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

About This Document

This document explains every component of the pipeline, from the high-level idea down to every tuneable number, written so that a newcomer to robotics can follow along. It covers the complete six-phase navigation system: from global path planning through obstacle avoidance to final wheel commands.

SECTION 1: THE BIG PICTURE — WHAT THIS SYSTEM DOES

1.1 THE PROBLEM WE ARE SOLVING

Multi-robot navigation is hard because robots need to reach goals without hitting each other, and they face three distinct types of obstacles:

• Static obstacles (walls, pillars): Known from the map, fixed positions that never move

• Dynamic cooperative obstacles (other robots): We know exactly where they are and they follow the same rules

• Dynamic uncooperative obstacles (humans, rolling objects): We see them on camera but cannot measure their speed reliably

Why you cannot use a single algorithm for all three types? Imagine driving on a highway. You know where the road barriers are (static). You can coordinate with other self-driving cars via V2V radio (cooperative swarm). But a pedestrian walking unexpectedly is an unknown — you have no radio link and their speed estimate is noisy.

1.2 THE SOLUTION: A PIPELINE OF SPECIALISTS

The DAG (Directed Acyclic Graph) concept means data flows in one direction, like an assembly line. Each stage has exactly one job. No stage looks backwards. This makes the system predictable and easy to debug.

The pipeline flowchart:

Hybrid A* → Waypoint Tracker → APF (Shield) → Intention Blender

↓

NH-ORCA (Scalpel) → Kinematic Mapper → Wheels

1.3 THE ARCHITECTURAL FIREWALL — THE MOST IMPORTANT RULE

APF never sees swarm agents. ORCA never sees camera blobs.

Why this separation is essential:

• If ORCA receives noisy velocity estimates (from camera frame differencing), its Velocity Obstacle cones swing violently every frame → robot stutters

• If APF processes swarm agents, it will fight ORCA's decisions → deadlocks

• The ONLY communication between APF and ORCA is through V_pref = V_path + F_unknown in the Intention Blender

Input/Output separation:

| Module | What to ALWAYS pass | What to NEVER pass |
| --- | --- | --- |
| APF (camera_blobs) | Unknown obstacle centroids from overhead camera (humans, objects) | Swarm robot positions |
| NH-ORCA (neighbors) | Perfect telemetry of other swarm robots | Camera blob positions or velocities |
| GlobalPath | A* waypoints around static map obstacles | Robot positions, camera data |
| WaypointTracker | Robot odometry position only | Obstacle data of any kind |

SECTION 2: PHASE 1 — GLOBAL PLANNER (Hybrid A*)

2.1 WHAT IT DOES

Runs ONCE at startup. Finds the shortest path around known static architecture (walls, U-shapes, pillars). Outputs a list of (x, y) waypoints.

2.2 WHY IT IS BLIND TO OTHER ROBOTS

The A* planner ignores all robots and camera data. If it didn't, the search tree would need to account for every possible robot position — exponentially larger. By keeping it static-only, planning stays at O(1) per robot regardless of swarm size.

2.3 THE GlobalPath CLASS

From integration.py, the GlobalPath class provides:

• set_path(waypoints): loads the A* result — each waypoint is (x, y) in metres

• set_straight_line_path(start, goal, n_steps): stub for testing without A*

• is_loaded property: returns True if path has been set

2.4 HOW TO CONNECT YOUR OWN A* PLANNER

Plug in your Hybrid A* output using set_path() with a list of (x, y) tuples in metres, in traversal order:

path.set_path([(0.0, 0.0), (1.0, 0.5), (2.0, 0.0), (3.0, 0.0)])

SECTION 3: PHASE 2 — WAYPOINT TRACKER ("The Floating Carrot")

3.1 THE STALLING PROBLEM AND THE CARROT SOLUTION

If the robot must hit waypoint N to within 1 cm before moving to N+1, any slight drift from odometry means it circles forever. The floating carrot fixes this: instead of requiring exact arrival, we scan a window of upcoming waypoints, find the closest one to the robot RIGHT NOW, then target a point K steps AHEAD of that. The carrot moves forward automatically as the robot progresses.

3.2 THE ALGORITHM STEP BY STEP

Step 1: Scan indices [current_idx, current_idx + lookahead_window]

Step 2: Find the waypoint in that window with minimum Euclidean distance

Step 3: Update current_idx to that closest waypoint's index

Step 4: Target the waypoint at index current_idx + carrot_steps

Step 5: Compute velocity vector: direction × min(max_speed, distance)

Step 6: If distance to FINAL waypoint < goal_tolerance → declare goal reached

3.3 TUNEABLE PARAMETERS TABLE

| Parameter Name | Default | Units | Effect of Increasing | Effect of Decreasing |
| --- | --- | --- | --- | --- |
| max_speed | 0.26 | m/s | Faster goal reach; less reaction time | Slower but safer |
| lookahead_window | 15 | waypoints | More forgiving; can skip waypoints | Tighter path following |
| carrot_steps | 3 | waypoints | Smoother arcs; may cut corners | Tighter turns; risk of stalling |
| goal_tolerance | 0.15 | m | Declares goal reached earlier | More precise; risk of stalling |

3.4 OUTPUT: V_PATH

V_path is a 2D velocity vector [vx, vy] in metres per second pointing from the robot's current position toward the carrot waypoint. Its magnitude equals min(max_speed, distance_to_carrot). It has no knowledge of obstacles — that is handled by the downstream stages.

SECTION 4: PHASE 3 — THE SHIELD (Artificial Potential Fields)

4.1 THE CORE METAPHOR

Imagine the robot lives on a hilly terrain. The goal is a deep valley — gravity pulls the robot toward it. Every obstacle is a sharp mountain — the robot is pushed away. At every moment, the robot feels the combined slope of this landscape and slides in that direction. This is all APF does: compute the slope at the robot's current position.

4.2 WHY APF IS PERFECT FOR UNKNOWN OBSTACLES

ORCA NEEDS to know an obstacle's velocity to compute the collision cone. If the overhead camera tracks a person frame-by-frame and differentiates position, the estimated velocity is noisy — it flickers left and right every 16ms. Feed that into ORCA and the cone swings wildly → robot stutters. APF does not care. It sees: 'there is a blob at (5, 5)'. Next frame: 'blob moved to (5, 6)'. The repulsion mountain smoothly shifts. The robot smoothly curves away regardless of the blob's speed.

4.3 THE MATHEMATICS — ATTRACTION

The Goal Pull (Attractive Force):

F_att = -k_att × (robot_pos - goal_pos)

Each term explained:

• k_att: the strength of the pull (bigger = chases goal more aggressively)

• (robot_pos - goal_pos): vector FROM goal TO robot

• The negative sign: flips it so the force points FROM robot TOWARD goal

• Why LINEAR and not quadratic: linear gives constant pull speed regardless of distance. Quadratic pulls harder far away but weaker near goal, causing overshoot and oscillation.

IMPORTANT: In this pipeline, get_repulsive_only() is used, NOT get_force(). The attractive pull is handled by the waypoint tracker (V_path). If APF also added attraction, the goal would be double-counted.

4.4 THE MATHEMATICS — STATIC REPULSION

Repulsion from Walls and Pillars (Static Obstacles):

magnitude = k_rep × (1/ρ - 1/ρ₀) × (1/ρ²)

direction = unit_vector(robot_pos - closest_surface_point)

Each term explained:

• ρ (rho): distance from the robot to the SURFACE of the obstacle

• ρ₀ (rho_0): the influence radius — if ρ > ρ₀, force = 0, obstacle ignored

• (1/ρ - 1/ρ₀): goes to zero as ρ → ρ₀ (smooth fade-in), blows up as ρ → 0

• (1/ρ²): makes the force stronger the closer you get (inverse square law)

• k_rep: overall repulsion gain

The max_force cap (default 5.0 m/s): Without a cap, when ρ → 0 (robot nearly touching wall), the formula gives infinity. This prevents the math singularity from causing the robot to teleport or spin.

4.5 THE MATHEMATICS — DYNAMIC REPULSION

Repulsion from Unknown Camera Blobs (Dynamic Obstacles):

Three key differences from static repulsion:

Difference 1 — Surface distance: ρ = dist(robot, blob_centre) - blob_radius. We subtract the blob's radius so the repulsion is measured from the SURFACE of the detected object, not its tracking centroid. Safety floor: ρ is clamped to minimum 0.05 m to prevent division by zero.

Difference 2 — Priority multiplier: k_eff = k_rep × obs.priority. A blob with priority=2.0 gets twice the push. Use this when the camera detects a large fast-moving object.

Difference 3 — Vortex (tangential) force:

F_vortex = (magnitude × vortex_gain) × perpendicular(radial_direction)

The radial force alone creates a symmetric deadlock: if robot and blob are heading exactly toward each other, the push is perfectly backward — net velocity = zero. The vortex force adds a 90° tangential component, causing the robot to curve LEFT around the obstacle. This breaks symmetry consistently.

4.6 TUNEABLE PARAMETERS TABLE

| Parameter | Default | Units | What it Controls |
| --- | --- | --- | --- |
| k_att | 1.0 | — | Strength of goal attraction. Rarely needs changing. |
| k_rep | 2.0 | — | Strength of all repulsion. Increase in cluttered environments. |
| rho_0 | 1.5 | m | Influence radius. Obstacles beyond this are ignored. |
| max_force | 5.0 | m/s | Hard cap on repulsion. Prevents singularity blowup. |
| vortex_gain | 0.4 | — | Tangential spin fraction. Higher = wider arc. 0 = no vortex. |

4.7 THE DynamicObstacle DATA CLASS

From apf.py, DynamicObstacle fields:

• x, y: camera centroid in world coordinates (after coordinate transform)

• radius: estimated from bounding box diagonal / 2.0

• priority: set by the detection pipeline based on blob size or persistence

4.8 WHAT APF RETURNS: F_UNKNOWN

get_repulsive_only() returns a 2D vector [fx, fy] in m/s units. It pushes the robot away from every active camera blob, weighted by distance and priority. This vector is added to V_path in the Intention Blender. If no blobs are detected, F_unknown = [0, 0] and V_pref = V_path unchanged.

SECTION 5: PHASE 4 — INTENTION BLENDING

5.1 THE FORMULA

V_pref = V_path + F_unknown

What this means intuitively: V_path says 'I want to go north-east to follow my A* route.' F_unknown says 'There is something dangerous to my south, push me north.' V_pref = north-east + north = slightly more northward curve. The robot bends its intended path around the unknown obstacle WITHOUT needing to know what the obstacle is or where it is going.

5.2 THE FIREWALL IN CODE

Exact lines from integration.py where the blending happens:

v_pref = self.blender.compute(

robot_pos = tuple(pos),

v_path = v_path,

camera_blobs = camera_blobs,  # ONLY unknown obstacles → APF

)

The NH-ORCA call that follows:

v_safe_v, v_safe_w = self.orca.update(

...

pref_vel = tuple(v_pref),

neighbors = neighbors,  # ONLY swarm agents → ORCA

)

camera_blobs and neighbors are NEVER passed to the wrong module. This is the firewall in action.

5.3 SPEED CLIPPING

max_pref_speed = 0.26 m/s: After blending, V_pref might exceed the robot's maximum speed if both V_path and F_unknown point in the same direction. We clamp the magnitude to max_pref_speed while preserving the direction. The robot is told 'you want to go this fast in this direction — but your hardware tops out here.'

SECTION 6: PHASE 5 — THE SCALPEL (NH-ORCA)

6.1 WHAT ORCA IS (PLAIN ENGLISH INTRODUCTION)

Imagine two people walking toward each other in a narrow hallway. Each person shifts slightly to their right — instinctively. Nobody announced this. Nobody communicated. Both assumed the other would also shift. ORCA is this instinct, formalized into math. Each robot independently calculates: 'If I stay on course, I will hit Robot B in τ seconds. If Robot B is also running ORCA, it will also shift. So I only need to shift HALF the required amount, and the collision disappears.' This is the 50/50 reciprocity that makes ORCA deadlock-free.

6.2 STANDARD ORCA vs NH-ORCA — THE CRITICAL DIFFERENCE

Standard ORCA assumes robots can slide in any direction instantly (holonomic motion). A differential-drive robot is NOT holonomic — it must turn first, then drive. NH-ORCA's fix has three parts:

1. Inflate the robot's collision radius by ε (tracking error)

2. Restrict allowed velocities to what the robot can physically track (P_AHV polygon)

3. Map the output velocity to real (v, ω) commands using kinematic equations

6.3 PART 1 — RADIUS INFLATION

ε (tracking_error = 0.05 m by default): A diff-drive robot following a curved path will always deviate slightly from the ideal holonomic path. By making every robot look 5 cm bigger in the ORCA math, we reserve that extra space. The real robot fits inside the inflated bubble.

r_inflated = robot_radius + tracking_error = 0.22 + 0.05 = 0.27 m

6.4 THE VELOCITY OBSTACLE (VO) — THE CORE CONCEPT

The Velocity Obstacle is a cone in velocity space — not position space. Here is what that means:

Step 1: Think in relative velocity space. v_rel = v_A - v_B

Step 2: Draw the forbidden cone. If v_rel points INTO the combined-radius disc centred at (robot B position minus robot A position), a collision will happen within τ seconds.

Step 3: Find the escape vector u. If the current v_rel is inside the cone, find the SMALLEST possible change to v_rel that escapes the cone.

Step 4: Split responsibility 50/50. Robot A only adjusts by (1/2)u. Robot B also adjusts by (1/2)u. Net relative change = u.

Step 5: Create the half-plane constraint. All velocities for Robot A that satisfy its half of the adjustment form a half-plane in velocity space.

Key formulas:

VO^τ_{A|B} = { v_rel | ∃t ∈ [0,τ] : t·v_rel ∈ D(p_B - p_A, r_A + r_B) }

ORCA half-plane:

ORCA^τ_{A|B} = { v | (v - (v_A^opt + c·u)) · n ≥ 0 }

where c = 0.5 for swarm agents (equal responsibility), c = 1.0 for static obstacles (ego avoids entirely)

6.5 THREE COLLISION DETECTION CASES

Case 1 — Already overlapping (dist < r_A + r_B): Emergency! Robots are already touching. The normal VO math doesn't apply. The normal vector points straight away from the other robot. Full velocity change is applied to push out immediately.

Case 2 — Relative velocity inside the VO cone: Collision predicted within τ seconds. Find escape via truncation disc (close range) or cone leg (longer range).

Case 3 — Relative velocity outside the VO cone: No collision predicted. Return None (no constraint). Robot continues on its preferred velocity for this pair.